# Fortigate

Microsoft Azure Test Drive

Test Drive Guide

Updated: December 15th, 2021

TAble of
Contents

Fortigate Test Drive	3

How to use this guide	3

About the test drive environment	3

TASK 1 – Connect to FortiGate & Webserver	4

TASK 2 – Add outbound connectivity Policy	8

TASK 3 – Install Apache2 Webserver	10

TASK 4 – Configure FortiGate for web traffic	10

TASK 5 – Connect to Webserver	13

# Fortigate
Test Drive

This test drive will allow you to experience how a FortiGate firewall enables enterprises to control their resources and applications in Microsoft Azure through Fortinet solutions. Fortinet solutions provide the freedom to deploy any application on Microsoft Azure without compromising security.

## How to use this guide

The activities outlined in this test drive guide contain all the information necessary to complete the defined scenarios and outlined tasks. Only a web browser is required to complete the test drive.

## About the test drive environment

You will configure Firewall policies on the FortiGate-VM firewall via the FortiGate GUI to install and enable a webserver hosted in Azure access to the Internet and then enable Virtual IPs to provide secured access from your device to the webserver hosted in Azure and protected by the FortiGate.

## TASK 1 – Connect to FortiGate & Webserver

- When the Test Drive is ready click on the FortiGate link to open the GUI.

- Use the following credentials

username: ftnt-testdrive
password: Fortinet@123

- Click the Later button to bypass FortiGate Dashboard setup

- Click Ok to bypass “What’s New in FortiOS 7.0”

- Open the FortiGate CLI via the FortiGate GUI

Click on the CLI Console Icon

- Connect to the webserver host via the CLI Console

exec ssh ftnt-testdrive@10.0.3.4
password: Fortinet@123

- Attempt to update apt to install Apache2 webserver

sudo apt-get update

The webserver host cannot connect to the Internet and will stall attempting to update the apt package repositories.

This is because an Azure route table with a User Defined Route has been added to the VNET to force the webserver host’s outbound communication through the FortiGate, and the FortiGate does not have a policy to allow internet connectivity.

- Minimize the CLI Console.

## TASK 2 – Add outbound connectivity Policy

- Select “Policy & Objects” -> “Firewall Policy”

- Click the “+ Create New” button

- Configure the Policy

  - Name: AllowOutBound

  - Incoming Interface: port2

  - Outgoing Interface: port1

  - Source: all

  - Destination: all

  - Schedule: always

  - Service: HTTP & HTTPS

  - Enable: NAT

  - Click OK

## TASK 3 – Install Apache2 Webserver

- Maximize the CLI Console session

- Attempt to install the Apache2 webserver

sudo apt-get update
sudo apt-get install apache2 -y

This time apt-get should update and the Webserver should install successfully, because the outbound traffic was allowed to pass through the FortiGate.

- Close the CLI Console

## TASK 4 – Configure FortiGate for web traffic

- In a new tab in your web browser, attempt to connect via http to the same public IP as the FortiGate.

This will not be successful because the FortiGate is not configured to respond to port 80.

- In the FortiGate GUI select “Policy & Objects” -> “Virtual IPs”

- Click the “+ Create New” button and select “Virtual IP”

- Create a new virtual IP to forward traffic for interface “port1”

  - Name: WebTrafficToWebserver

  - Interface: port1

  - External IP Address/Range: 10.0.1.4

  - Mapped IP Address/Range: 10.0.3.4

  - Enable Port Forwarding

  - External Service Port: 80

  - Map to Port: 80

  - Click OK

- Select “Policy & Objects” -> “Firewall Policy” You should see the AllowOutBound policy that was previously created.

- Click the “+ Create New” button.

- The new policy will allow all traffic in port1 and out port2 (the reverse of the previous policy).

  - Name: WebTrafficToWebserverVIP

  - Incoming Interface: port1

  - Outgoing Interface: port2

  - Source: all

  - Destination: WebTrafficToWebserver

  - Service: HTTP

  - Click OK

## TASK 5 – Connect to Webserver

- Attempt to connect again to the public IP via http. This time you should see the default Apache2 for Ubuntu web page.

- Success!

This Azure FortiGate Test Drive is a simple use case that enables hosts in a protected subnet the ability to access the Internet via the FortiGate and allow external clients access to resources in a protected subnet via the FortiGate.

Whether the traffic is outbound or inbound it can be monitored and managed by the FortiGate allowing for secured network communications.